# P2-iST SOP on Opentrons Flex

# (36-96 samples)

This protocol is written to process up to 100 µl plasma using the Preomics P2-iST kit. All reagents are included in the P2-iST kit. This Opentrons protocol does not include the Purify section (iST cleanup) of the P2-iST protocol. Protocol name on Opentrons Flex: P2-iST

# Variable Parameters

| Name | Default | Range |
| --- | --- | --- |
| Run Type | Sample Run | Sample Run, Water Run, Dry Run |
| Sample Column count (8 samples per column) | 12 | 3-12 |
| Multi-channel Mount | Right | Right, Left |

# Fixed Parameters

| Name | Default |
| --- | --- |
| Run Type | Sample Run |
| Lyse, Reduction, Alkylation | Included |
| Incubation Temperature | 37°C |
| Incubation time | 1 hr |
| Reagent Plate type | Large 250 µL Thermo |

# Materials

- Opentrons Flex 96 Filter Tip Rack 200 µL (6 is the maximum, put at least X for XX samples) Deck Slots B1, B2, B3, A4, B4, C4, D4

- Opentrons Flex 96 Filter Tip Rack 50 µL Deck Slots A1, A2 (not necessary?)

- Magnetic Block  Deck Slot C1

- Universal HeatShaker Adaptor (flat, rectangular) change adaptors with purple star shaped screwdriver  Deck Slot D1

- Opentrons Tough 96 Well Plate 200 µL PCR Full Skirt  samples (100 µL plasma) Deck Slot D1 (on heater shaker)

- Empty ThermoFisher 96 well plate 250 µL  final peptides  Deck Slot A3

- NEST 96 Deep Well plate 2 mL  Deck Slot D4

  - Robot moved – P2 Bind Buffer  Column 1

  - Robot moved – P2 Beads  Column 2

  - Robot moved – Lyse mix  Column 3

  - Robot moved – Stop solution  Column 5

- Opentrons 24 Tube Rack  Bind buffer, Beads, Lyse buffer, Digest-L, Stop buffer  Deck Slot C3

- NEST 12 Well Reservoir 15 mL  wash buffer  Deck Slot C2

# Notes

- PIPETTES  LEFT: 1-channel 1000uL      RIGHT: 8-channel 1000uL

- Samples up to 100µL volume in Deck Slot D1 (Opentrons Touch 96 well plate 200 µL PCR Full Skirt).

- P2 beads are placed in tube A1 (and A2 if columns>6) of Deck Slot C3.  Beads should be washed before

- Solutions Lyse A and Lyse B are mixed before

PAUSE point Once the lysate-bead is incubating, set your timer to 25 min. In the last 5 minutes, prepare your trypsin solution/digestion buffer. Once complete, hit pause on the Robot and place falcon tube in Slot C3, position A3.

Make sure to double check that everything is in the correct position. Refer to layout on next page.

If labware offsets are not present, perform a labware position check with the calibration probe to ensure no crashes occur.

C3 - Opentrons 24 Tube Rack -  2 mL tubes

P2 Bind (2 mL tubes)  75 µl/sample, provide (75+3)*8*COLUMNS + dead volume (15 µL per tube)

- 1.825 mL per tube for each 3 columns (or 24 samples) = 4 tubes needed for 96 samples

P2 Beads (2 mL tubes)  25 µl/sample, provide (25+3)*8*COLUMNS + dead volume (15 µL per tube)

- 1.225 mL per tube for each 6 columns (or 48 samples) = 2 tubes needed for 96 samples

Lyse A/B mix (2 mL tube) – 40 µL/sample, provide (40+3)*8*COLUMNS + dead volume (15 µL per tube)

- 0.980 mL per tube for each 3 columns (or 24 samples) = 4 tubes needed for 96 samples

Digest-L resuspended (2 mL tube)  10 µL/sample, provide 10*8*COLUMNS + 25 µL

Stop buffer (2 mL tube) – 60 µL/sample, provide 60*8*COLUMNS + dead volume (25 µL per tube)

- 1.465 mL per tube for each 3 columns (or 24 samples) = 4 tubes needed for 96 samples

C2 – NEST 12 well reservoir 15 ml

Wash Buffer  600 µL/sample, provide 600*sample#+1500 µL

- 11 mL per well for each 2 columns (or 16 samples) = 6 wells needed for 96 samples

NEST 12 Deep well plate 15 ml

Thermofisher 96 well plate 250 ul

P50 tips (filter)

P200 tips (filter)

Magnetic Block

NEST 96 Deep well plate 2 ml

Universal Heat shaker adaptor

Samples